//compass.h

#ifndef COMPASS_H

#define COMPASS_H

#include <Arduino.h>

#include <QMC5883LCompass.h>

#include <Preferences.h>

#include "pin_manager.h"

class Compass

{

private:

QMC5883LCompass qmc5883l;

Preferences memory;

public:

float off_x, off_y, off_z;

float scale_x, scale_y, scale_z;

Compass();

void begin();

void start_calibrate();

void save_data();

void load_data();

void check_calibrate();

int get_heading();

const char *store = "MyCompass_data"; // namespace Flash

const char *key_used = "useCount";

const char *key_flag = "calibrated";

};

#endif

//detect_rssi.h

#ifndef DETECT_RSSI_H

#define DETECT_RSSI_H

#include <Arduino.h>

#include <WiFi.h>

class detect_rssi

{

private:

const char* ssid;

const char* password;

public:

detect_rssi();

void begin();

long get_rssi();

void stop();

};

#endif

//display.h

#ifndef DISPLAY_H

#define DISPLAY_H

#include <U8g2lib.h>

#include <Arduino.h>

#include "pin_manager.h"

class oled_display {

private:

U8G2_SSD1306_128X64_NONAME_F_HW_I2C u8g2;

const int center_x = 64;

const int center_y = 32;

const int max_radius = 28;

void draw_compass_labels();

void draw_heading_arrow(int heading);

void draw_wifi_dot(int best_heading, long rssi);

public:

oled_display();

void begin();

void clear();

void show_wifi_status(bool connected);

void render_interface(int heading, int best_heading, long rssi);

};

#endif

#ifndef MOTOR_DRIVER_H

#define MOTOR_DRIVER_H

#include <Arduino.h>

#include "pin_manager.h"

class motor_driver

{

public:

motor_driver();

void begin();

void move_forward(int speed);

void move_backward(int speed);

void turn_left(int speed);

void turn_right(int speed);

void stop();

};

#endif

//Navigation.h

#ifndef NAVIGATION_H

#define NAVIGATION_H

#include <Arduino.h>

#include "detect_rssi.h"

#include "Compass.h"

class navigation

{

private:

detect_rssi* rssi_store;

Compass* compass_store;

int best_rssi ;

int best_heading ;

public:

navigation(detect_rssi* rssiPtr, Compass* compassPtr);

void begin();

void update_best_direction();

int get_best_heading();

long get_current_rssi();

int get_current_heading();

};

#endif

#ifndef PIN_MANAGER_H

#define PIN_MANAGER_H

#include<Wire.h>

#include <HardwareSerial.h>

//  Cấu hình chân dùng chung

struct PinManager {

// I2C cho QMC5883L + MPU6050 + OLED

static const int I2C_SDA = 3;

static const int I2C_SCL = 2;

//motor

static const int in1 = 8;

static const int in2 = 7;

static const int in3 = 6;

static const int in4 = 5;

static void init_i2c()

{

Wire.begin(I2C_SDA, I2C_SCL);

}

};

#endif

//compass.cpp

#include "Compass.h"

Compass::Compass() {}

void Compass::begin()

{

qmc5883l.init();

qmc5883l.setSmoothing(10, false);

check_calibrate();

qmc5883l.setCalibrationOffsets(off_x, off_y, off_z);

qmc5883l.setCalibrationScales(scale_x, scale_y, scale_z);

}

void Compass::start_calibrate()

{

qmc5883l.calibrate();

off_x = qmc5883l.getCalibrationOffset(0);

off_y = qmc5883l.getCalibrationOffset(1);

off_z = qmc5883l.getCalibrationOffset(2);

scale_x = qmc5883l.getCalibrationScale(0);

scale_y = qmc5883l.getCalibrationScale(1);

scale_z = qmc5883l.getCalibrationScale(2);

}

void Compass::save_data()

{

memory.begin(store, false);

memory.putFloat("off0", off_x);

memory.putFloat("off1", off_y);

memory.putFloat("off2", off_z);

memory.putFloat("scale0", scale_x);

memory.putFloat("scale1", scale_y);

memory.putFloat("scale2", scale_z);

memory.end();

Serial.println("Success save calibrate ");

}

void Compass::load_data()

{

memory.begin(store, true);

off_x = memory.getFloat("off0");

off_y = memory.getFloat("off1");

off_z = memory.getFloat("off2");

scale_x = memory.getFloat("scale0");

scale_y = memory.getFloat("scale1");

scale_z = memory.getFloat("scale2");

memory.end();

}

void Compass::check_calibrate()

{

memory.begin(store, false);

bool isCalibrated = memory.getBool(key_flag, false);

int useCount = memory.getInt(key_used, 0);

if (!isCalibrated || useCount >= 3)

{

Serial.println("please move and wait to calibrate");

start_calibrate();

save_data();

memory.putBool(key_flag, true);

memory.putInt(key_used, 1);

Serial.println("Finishing calibrate");

}

else

{

load_data();

memory.putInt(key_used, useCount + 1);

Serial.printf("Number of use: %d\n", useCount + 1);

}

memory.end();

}

int Compass::get_heading()

{

qmc5883l.read();

int azimuth = qmc5883l.getAzimuth();

if (azimuth < 0)

{

azimuth += 360; // Chuyển đổi góc âm sang khoảng 180-359

}

Serial.print(" Azimuth : ");

Serial.println(azimuth);

return azimuth;

}

//dectect_rssi.cpp

#include "detect_rssi.h"

detect_rssi::detect_rssi()

{

ssid = "Huu yeu dau";

password = "pehuuyeudau";

}

void detect_rssi::begin()

{

WiFi.begin(ssid, password);

Serial.println("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println("Connected to WiFi");

}

long detect_rssi::get_rssi() {

long rssi = WiFi.RSSI();

Serial.print("RSSI: ");

Serial.print(rssi);

Serial.println(" dBm");

return rssi;

}

void detect_rssi::stop()

{

WiFi.disconnect();

}

//display.cpp

#include "display.h"

#include <math.h>

oled_display::oled_display()

: u8g2(U8G2_R0, /* reset=*/ U8X8_PIN_NONE) {}

void oled_display::begin() {

u8g2.begin();

u8g2.setFont(u8g2_font_6x12_tr);

}

void oled_display::clear() {

u8g2.clearBuffer();

u8g2.sendBuffer();

}

void oled_display::show_wifi_status(bool connected) {

u8g2.clearBuffer();

u8g2.setCursor(0, 32);

if (connected) {

u8g2.print("WiFi connected!");

} else {

u8g2.print("Connecting to WiFi...");

}

u8g2.sendBuffer();

}

// 🧭 Vẽ 4 chữ hướng: N/E/S/W

void oled_display::draw_compass_labels() {

u8g2.setCursor(center_x - 3, 10);  u8g2.print("N");

u8g2.setCursor(center_x - 3, 63);  u8g2.print("S");

u8g2.setCursor(0, center_y + 4);   u8g2.print("W");

u8g2.setCursor(120, center_y + 4); u8g2.print("E");

}

// 🠗 Vẽ mũi tên hướng hiện tại

void oled_display::draw_heading_arrow(int heading_deg) {

// Đảo chiều để khớp hệ tọa độ màn hình

float heading_rad = (360 - heading_deg) * DEG_TO_RAD;

int arrow_length = max_radius - 8;  // 🠗 Mũi tên ngắn hơn

int arrow_x = center_x + arrow_length * cos(heading_rad);

int arrow_y = center_y - arrow_length * sin(heading_rad);

u8g2.drawLine(center_x, center_y, arrow_x, arrow_y);

u8g2.drawDisc(center_x, center_y, 2); // Vẽ tâm xe

}

// 📡 Vẽ dấu chấm tín hiệu WiFi

void oled_display::draw_wifi_dot(int best_heading_deg, long rssi) {

float best_rad = (360 - best_heading_deg) * DEG_TO_RAD;

float strength_ratio = constrain(map(rssi, -90, -30, 0, 100), 0, 100) / 100.0;

int signal_radius = max_radius * strength_ratio;

// Nếu vượt quá bán kính thì neo vào rìa

if (signal_radius > max_radius) signal_radius = max_radius;

int dot_x = center_x + signal_radius * cos(best_rad);

int dot_y = center_y - signal_radius * sin(best_rad);

// Nhấp nháy theo chu kỳ thời gian (mỗi 500ms)

if ((millis() / 500) % 2 == 0) {

u8g2.drawDisc(dot_x, dot_y, 2);

}

}

// ✨ Vẽ toàn bộ giao diện

void oled_display::render_interface(int heading, int best_heading, long rssi) {

u8g2.clearBuffer();

draw_compass_labels();

draw_heading_arrow(heading);

draw_wifi_dot(best_heading, rssi);

u8g2.sendBuffer();

}

#include "Motor_control.h"

motor_driver::motor_driver(){}

void motor_driver::begin()

{

pinMode(PinManager::in1, OUTPUT);

pinMode(PinManager::in2, OUTPUT);

pinMode(PinManager::in3, OUTPUT);

pinMode(PinManager::in4, OUTPUT);

stop(); // đảm bảo mức 0

}

void motor_driver::move_forward(int speed) {

analogWrite(PinManager::in1, speed);

analogWrite(PinManager::in2, 0);

analogWrite(PinManager::in3, speed);

analogWrite(PinManager::in4, 0);

}

void motor_driver::move_backward(int speed) {

analogWrite(PinManager::in1, 0);

analogWrite(PinManager::in2, speed);

analogWrite(PinManager::in3, 0);

analogWrite(PinManager::in4, speed);

}

void motor_driver::turn_left(int speed) {

analogWrite(PinManager::in1, 0);

analogWrite(PinManager::in2, speed);

analogWrite(PinManager::in3, speed);

analogWrite(PinManager::in4, 0);

}

void motor_driver::turn_right(int speed) {

analogWrite(PinManager::in1, speed);

analogWrite(PinManager::in2, 0);

analogWrite(PinManager::in3, 0);

analogWrite(PinManager::in4, speed);

}

void motor_driver::stop() {

analogWrite(PinManager::in1, 0);

analogWrite(PinManager::in2, 0);

analogWrite(PinManager::in3, 0);

analogWrite(PinManager::in4, 0);

}

//Navigation.cpp

#include "Navigation.h"

navigation::navigation(detect_rssi* rssi_data, Compass* compass_data) {

rssi_store = rssi_data;

compass_store = compass_data;

}

void navigation::begin() {

Serial.println(" Navigation system ready.");

if (!compass_store) //kiểm tra con trỏ có hợp lệ không

{

Serial.println(" Compass module missing!");

} else {

compass_store->begin();

}

if (!rssi_store)

{

Serial.println("RSSI module missing!");

} else {

rssi_store->begin();

}

best_rssi = -999;

best_heading = 0;

Serial.println(" Modules initialized & state reset.");

}

void navigation::update_best_direction()

{

long current_rssi = rssi_store->get_rssi();

int current_heading = compass_store->get_heading();

if(current_rssi > best_rssi) // xoayy

{

best_rssi = current_rssi;

best_heading = current_heading;

Serial.println("Update best direction");

Serial.print("Best RSSI: ");

Serial.println(best_rssi);

Serial.print("Best Heading: ");

Serial.println(best_heading);

}

}

int navigation::get_best_heading() {

return best_heading;

}

long navigation::get_current_rssi() {

return rssi_store->get_rssi();

}

int navigation::get_current_heading() {

return compass_store->get_heading();

}